# Topic 4 Lesson 1 Quiz: Bits, bases and negative numbers

Work each answer out on paper first, then choose. No notes.

1. Which statement describes a word?

a. A group of exactly 8 bits

b. The number of bits the CPU can manipulate as a single unit

c. The smallest unit of data a computer can store

d. A group of 4 bits, shown as one hex digit

ANS: b

2. How many different values can one byte represent?

a. 256

b. 255

c. 128

d. 8

ANS: a

3. A computer has a 64-bit word length. How many bytes are in one word?

a. 64

b. 4

c. 8

d. 16

ANS: c

4. What is 10110100₂ in denary?

a. 164

b. 45

c. 184

d. 180

ANS: d

5. Which is 87₁₀ as an unsigned 8-bit binary number?

a. 01010111

b. 01011011

c. 11101010

d. 01010110

ANS: a

6. Which is 2A₁₆ in 8-bit binary?

a. 10100010

b. 00101100

c. 00101011

d. 00101010

ANS: d

7. What is 11001000₂ in hexadecimal?

a. 8C

b. C8

c. D8

d. 128

ANS: b

8. What is 7E₁₆ in denary?

a. 714

b. 112

c. 126

d. 127

ANS: c

9. What is 214₁₀ in hexadecimal?

a. 6D

b. C6

c. D6

d. 136

ANS: c

10. Why do programmers often write values in hexadecimal instead of binary?

a. The CPU processes hexadecimal faster than binary

b. It is shorter and easier for humans to read, with fewer copying errors

c. Hexadecimal values take up less memory

d. Binary cannot be used to store negative numbers

ANS: b

11. How is −39₁₀ stored using two's complement in an 8-bit register?

a. 10100111

b. 11011000

c. 00100111

d. 11011001

ANS: d

12. How is −20₁₀ stored using sign and magnitude in an 8-bit register?

a. 10010100

b. 11101100

c. 00010100

d. 11101011

ANS: a

13. An 8-bit register uses two's complement. What denary value does 11100101 represent?

a. 229

b. −101

c. −26

d. −27

ANS: d

14. What is the range of integers an 8-bit register can store using two's complement?

a. −127 to +127

b. 0 to 255

c. −128 to +127

d. −128 to +128

ANS: c

15. Which is a disadvantage of sign and magnitude compared with two's complement?

a. It has two different patterns for zero

b. It cannot store negative numbers

c. It needs more bits for the same range of positive numbers

d. Its most significant bit is worth −128

ANS: a

16. An 8-bit register uses sign and magnitude. What denary value does 10000001 represent?

a. −127

b. −1

c. 129

d. +1

ANS: b