# Lesson 2 End-of-Lesson Assessment

AS Computer Science – Masking, Registers and Boolean Laws

1. An 8-bit register contains 10110101. A programmer wants to test whether the most significant bit is 1. Which mask should be used with the AND operation?

A. 10000000

B. 00000001

C. 01111111

D. 11111111

ANS: A

2. A register containing 01101001 is ANDed with the mask 00001000. The result is 00001000. What does this show?

A. Bit 3 of the register is 0

B. Bit 3 of the register is 1

C. Bit 4 of the register is 1

D. Every bit of the register is 1

ANS: B

3. A register containing 10010110 is ANDed with the mask 00100000. The result is 00000000. Which statement is correct?

A. Bit 5 of the register is 1

B. The mask is not valid

C. Bit 5 of the register is 0

D. Every bit of the register is 0

ANS: C

4. Which operation and mask clear the rightmost 3 bits of an 8-bit register, leaving the other bits unchanged?

A. AND with 00000111

B. OR with 00000111

C. XOR with 11111000

D. AND with 11111000

ANS: D

5. A register containing 11011101 is ANDed with the mask 11111000. What is the result?

A. 11011000

B. 00000101

C. 11011101

D. 00011000

ANS: A

6. Which mask clears bits 2 and 5 only (counting from the right, starting at 0) and keeps every other bit?

A. 00100100

B. 11011011

C. 11011010

D. 10110111

ANS: B

7. When a mask is used with the AND operation, which statement is correct?

A. A 1 in the mask forces the bit to 1; a 0 keeps the original bit

B. A 1 in the mask flips the original bit; a 0 keeps it

C. A 1 in the mask keeps the original bit; a 0 forces the bit to 0

D. A 0 in the mask keeps the original bit; a 1 clears it

ANS: C

8. A masking question is worth three marks. Which set of answers earns all three?

A. The register, the mask and the truth table

B. The mask, the bit position and the register size

C. The operation, the truth table and the result

D. The mask, the operation (AND) and the result

ANS: D

9. Which Boolean law is shown by A . 0 = 0 ?

A. Annulment law

B. Identity law

C. Complement law

D. Idempotent law

ANS: A

10. Which Boolean law is shown by A + NOT A = 1 ?

A. Annulment law

B. Complement law

C. Involution law

D. Identity law

ANS: B

11. Which Boolean law states that (A + B) + C = A + (B + C) ?

A. Commutative law

B. Idempotent law

C. Associative law

D. Identity law

ANS: C

12. Which Boolean law states that NOT (NOT A) = A ?

A. Complement law

B. Annulment law

C. Idempotent law

D. Involution law

ANS: D

13. Which of the following is NOT a valid Boolean law?

A. A . A = 0

B. A + A = A

C. A . 1 = A

D. A + 0 = A

ANS: A

14. A student uses truth tables to prove a Boolean law. What must be true for the law to be proved?

A. Both sides contain the same number of variables

B. The columns for both sides are identical for every combination of inputs

C. The last row of each column is the same

D. Both sides contain the same number of 1s

ANS: B

15. A student builds a truth table to prove the associative law for the variables A, B and C. How many rows are needed?

A. 3

B. 6

C. 8

D. 9

ANS: C

16. Simplify: B . (A + NOT A)

A. 1

B. A + B

C. 0

D. B

ANS: D

17. In the simplification B . (A + NOT A) = B . 1 = B, which law justifies the first step?

A. Complement law

B. Identity law

C. Annulment law

D. Idempotent law

ANS: A

18. Simplify: A . A + B . 0

A. A + B

B. A

C. 0

D. B

ANS: B

19. Simplify: C + C . 1

A. 1

B. 0

C. C

D. NOT C

ANS: C

20. Which method is NOT accepted as a way of simplifying an expression in an exam question?

A. Naming the law used on each line

B. Applying one law per line

C. Showing every step of working

D. Using a truth table only

ANS: D