Exam practice: floating point

Topic 4, Lesson 3  (spec 4.4.3, 4.4.4)

| Name |  | Date |  |
| --- | --- | --- | --- |

How to use this sheet Type your answers in the shaded boxes. Floating point grids have one square per bit, with the place values above. Always write the mantissa, the exponent and the final answer: each one earns a mark. Mantissa and exponent use two’s complement throughout. About 45 minutes. Total: 37 marks.

Section A: Floating point to denary

1  In a certain computer system, real numbers are stored in normalised floating point form using two’s complementation, an 8 bit mantissa and a 3 bit exponent. The following is a floating point representation of a real number: 0.1110100 010. Calculate the denary value of the mantissa and exponent, and convert this floating point number into a denary number.	[3]

Past paper: 2022, Q4(d)(i)

|  | Mantissa (denary) | Exponent (denary) | Answer |
| --- | --- | --- | --- |
| Value |  |  |  |

Working

2  Using an 8-bit mantissa and a 4-bit exponent, both in two’s complement, convert 01011000 0101 into denary. Show the denary value of the mantissa and the exponent.	[3]

Practice

|  | Mantissa (denary) | Exponent (denary) | Answer |
| --- | --- | --- | --- |
| Value |  |  |  |

Working

Section B: Denary to floating point

3  In a certain computer system, real numbers are stored in floating point form using two’s complementation, a 12 bit mantissa and a 4 bit exponent. Convert the number 16.125₁₀ into this floating point form.	[3]

Past paper: 2018, Q5(b)(i)

Fixed point binary

| Mantissa |  |  |  |  |  |  |  |  |  |  |  |  | Exponent |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| −1 | ½ | ¼ | ⅛ | 1/16 | 1/32 | 1/64 | 1/128 | 1/256 | 1/512 | 1/1024 | 1/2048 |  | −8 | 4 | 2 | 1 |

4  Convert 6.375₁₀ into normalised floating point form using an 8-bit mantissa and a 4-bit exponent, both in two’s complement.	[3]

Practice

Fixed point binary, and how far the point moves

| Mantissa |  |  |  |  |  |  |  |  | Exponent |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| −1 | ½ | ¼ | ⅛ | 1/16 | 1/32 | 1/64 | 1/128 |  | −8 | 4 | 2 | 1 |

5  Convert 0.375₁₀ into normalised floating point form using an 8-bit mantissa and a 4-bit exponent, both in two’s complement.	[3]

Practice

Beyond the past papers: WJEC have not used negative exponents, but this shows how very small numbers are stored.

Fixed point binary, and which way the point moves

| Mantissa |  |  |  |  |  |  |  |  | Exponent |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| −1 | ½ | ¼ | ⅛ | 1/16 | 1/32 | 1/64 | 1/128 |  | −8 | 4 | 2 | 1 |

Section C: Range

6  Real numbers are stored in normalised floating point form using two’s complementation, a 6-bit mantissa and a 4-bit exponent.

Practice

(a)  Showing your workings, calculate the largest positive denary number that this system can store.	[3]

| Mantissa |  |  |  |  |  |  | Exponent |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| −1 | ½ | ¼ | ⅛ | 1/16 | 1/32 |  | −8 | 4 | 2 | 1 |

|  | Mantissa (denary) | Exponent (denary) | Answer |
| --- | --- | --- | --- |
| Value |  |  |  |

(b)  State the smallest positive normalised number this system can store, using an exponent of 0 as the lowest exponent.	[1]

7  In a certain computer system, real numbers are stored in normalised floating-point form using a positive 4 bit mantissa and a positive 4 bit exponent. Calculate the denary range of positive real numbers that can be stored in this normalised floating-point form. Show all your workings.	[6]

Past paper: 2019, Q11(b)

My assumption about the exponent (unsigned or two’s complement)

|  | Mantissa | Exponent | Denary value |
| --- | --- | --- | --- |
| Smallest |  |  |  |
| Largest |  |  |  |

Working

Section D: Extended response

8  Andrew is considering using three different methods for storing positive numbers, using two’s complementation, in a computer system. Method 1: store numbers as an integer using an 8 bit register. Method 2: store real numbers in normalised floating-point form, using a 5 bit mantissa and a 3 bit exponent. Method 3: store real numbers in normalised floating-point form, using a 4 bit mantissa and a 4 bit exponent. Calculate the denary range (highest and lowest) of numbers that can be stored by each method. Compare the advantages of representing numbers in integer and floating-point forms.	[12]

Past paper: 2023, Q14

|  | Highest (with working) | Lowest (with working) |
| --- | --- | --- |
| Method 1 |  |  |
| Method 2 |  |  |
| Method 3 |  |  |

Compare the advantages of integer and floating-point forms. Write in full sentences.

| My mark | / 37 | One thing I need to practise: |
| --- | --- | --- |