AS Computer Science  |  Unit 1  |  Topic 3: Data transmission

# Lesson 1 of 3: How data travels

Serial and parallel transmission; simplex, half duplex and full duplex

| Duration | 90 minutes | Specification | 3.1.1 and 3.1.2 |
| --- | --- | --- | --- |
| Group | AS Computer Science | Next lesson | Multiplexing, switching and packet contents (3.1.3, 3.2.1) |

## Learning objectives

By the end of the lesson students will be able to:

- Describe serial and parallel transmission (3.1.1)

- Explain the advantages and disadvantages of each, including skew and crosstalk (3.1.1)

- Describe simplex, half duplex and full duplex transmission (3.1.2)

- Apply each method to a real example, as the exam expects (3.1.2)

## Key terms  |  Termau allweddol

| English | Cymraeg (draft) | Meaning |
| --- | --- | --- |
| Serial transmission | Trosglwyddo cyfresol | Data sent one bit at a time along a single channel |
| Parallel transmission | Trosglwyddo paralel | Several bits sent at the same time, each on its own wire |
| Skew | Sgiw | Bits on different wires arriving at slightly different times |
| Crosstalk | Croes-siarad | Interference between wires running side by side |
| Simplex | Simplecs | Data travels in one direction only |
| Half duplex | Hanner deublyg | Data travels in both directions, but only one direction at a time |
| Full duplex | Deublyg llawn | Data travels in both directions at the same time |

Welsh terms are a draft. Check them against Y Termiadur Addysg before the Teams post goes out, especially skew and crosstalk.

## Resources

- PowerPoint: Topic3_L1_How_data_travels.pptx (24 slides)

- Starter: Which way does it travel? (self-paced HTML)

- Bit race: serial vs parallel (paired HTML interactive)

- Duplex detective (paired HTML interactive)

- Exam practice sheet with mark scheme (Word)

- Your existing quizzes: serial and parallel; simplex and duplex

- NotebookLM video summary for Topic 3

- Teams post with objectives, key terms, exit question and homework

## Lesson sequence

| Time | Stage | What happens | Resource |
| --- | --- | --- | --- |
| 0–10 | Starter | Students work through the self-paced sort on their own: six everyday examples into one way only, both ways taking turns, and both ways at once. Part 2: they write one sentence describing each group and add an example of their own. No technical terms yet. | Starter HTML; slide 4 |
| 10–15 | Video | NotebookLM summary. Students note one difference between serial and parallel, one reason parallel is used less today, and which group a phone call belongs to. | Slide 5 |
| 15–25 | Teach: serial and parallel | Definitions with diagrams: one bit at a time on a single channel vs several bits at once on separate wires. Where each is used. Why parallel struggles over distance: skew and crosstalk. Comparison table. Pose, but don’t answer: "So why is almost everything serial now?" | Slides 6–11 |
| 25–40 | Bit race (pairs) | Mission 1: parallel is about 8 times faster over short distances. Mission 2: at 100 m the parallel message corrupts (skew). Mission 3: serial can be clocked faster and beats parallel with no errors. Pairs then answer 2025 Q3(a) together before revealing the mark scheme. | Bit race HTML; slides 12–14 |
| 40–45 | Check | Serial and parallel quiz. | Your quiz; slide 15 |
| 45–55 | Teach: transmission methods | Simplex, half duplex and full duplex with the mark scheme examples (keyboard, walkie-talkie, broadband). Back to the starter: students relabel their groups with the technical terms and compare their descriptions with the definitions. Getting the marks: "both directions" alone does not earn the half duplex mark. | Slides 16–19 |
| 55–70 | Duplex detective (pairs) | 12 scenario cards. For each: choose the method, write a one-line justification, pick the explanation that would earn the mark, then judge their own justification against the model. Tricky cases: basic vs talk-back baby monitor, pilot and air traffic control, games controller, old network hub. | Duplex detective HTML; slide 20 |
| 70–75 | Check | Simplex and duplex quiz. | Your quiz; slide 21 |
| 75–87 | Exam practice | 2023 Q6 [6] on the exam practice sheet, 6 minutes, on their own. Fast finishers move on to 2016 Q5(a) [3]. Self-mark in a different colour using the mark scheme. | Exam practice sheet; slides 22–23 |
| 87–90 | Plenary | Exit question on Teams: "If parallel is faster, why is USB serial?" Two sentences using skew and clock speed. Set homework. | Slide 24; Teams post |

## Misconceptions to watch for

- "Serial is always slower." Older textbooks say this. Parallel moves more bits per clock tick, but skew and crosstalk limit its clock speed, so modern serial links (USB, SATA, PCIe) are faster overall.

- Mixing up serial with simplex. Serial and parallel are about how many bits travel at once. Simplex and duplex are about direction. Duplex detective uses this as a distractor.

- "Half duplex sends data both ways." True, but this alone doesn’t earn the mark. Students must say only one direction at a time.

- Benefits instead of descriptions. "Full duplex is faster" doesn’t describe the method, so it doesn’t earn the description mark.

## Differentiation

### Support

- The starter gives hints on each wrong placement and reveals the answers after the third check, so nobody stalls.

- Duplex detective step 4 gives three explanations to choose from rather than asking for one from scratch.

- Pair weaker students with a confident partner for both paired interactives.

### Stretch

- Bit race: ask which wire caused the corruption, and why a slower clock helps parallel but never fixes it at 100 m.

- Duplex detective: challenge pairs to argue why a USB keyboard is technically not simplex (Caps Lock signals come back).

- 2016 Q5(a) extension: rewrite the 6-mark answer for a 3-mark question.

## Assessment for learning

- Starter descriptions, compared with the definitions at slide 18

- Bit race answers to 2025 Q3(a), checked against the mark scheme in the tool

- Two quizzes at the end of each half

- Duplex detective scores and copied results

- 2023 Q6 self-marked against the mark scheme

- Exit question replies on Teams

## Past exam questions on this lesson’s content

| Series | Question | Topic | Marks |
| --- | --- | --- | --- |
| 2016 | Q5(a) | Describe simplex, half duplex and full duplex | 3 |
| 2023 | Q6 | Simplex, half duplex and full duplex with an example of each | 6 |
| 2025 | Q3(a) | Two advantages and two disadvantages of parallel over serial | 4 |

## Homework

Read section 3.1.3 (Multiplexing and switching) in the Topic 3 notes, then answer:

- What is multiplexing?

- Why is multiplexing needed on a network?

- Give one difference between circuit switching and packet switching.

Students who did not finish Duplex detective complete it and paste their results on the Teams post.